Mirror protector:

Function: Prevent the Glass mirror theft.

Material: Acrylonitrile Styrene Acrylate (ASA BLACK) / ABS

Texture: COD.001

Adhesive: Epoxy (EP-20/H-97M)

Fitment:

- Part should have at least 2mm of gap against all mirror adjustment positions.

- Should fit along all edges. Should not interfere with any mirror functionality.

Vehicle: B10 & C10/C16

P/N: 82219786AA

Weight: 0.21 kg

Pieces per set: 2# - 1 - LH & 1 - RH

Process: Injection Molding